Bytehound

Serial Telemetry & Decoder Guide

Framed UART & Modbus RTU telemetry · live plots · TX commands · offline analysis

Table of Contents

Right-click → 'Update Field' to populate the Table of Contents.

# 1. Introduction & Getting Started

## What is the Bytehound?

Bytehound is a hardware-agnostic telemetry and control dashboard for serial devices. It reads framed UART/USB byte streams or Modbus RTU traffic, decodes the payload using a workbook or CSV configuration that you define, and displays the results in live tables, plots, logs, and command panels — all without writing Python code.

### Application Data Flow & Architecture

Figure: Application Data Flow & Architecture

## Connecting Your Hardware

To get your hardware (NXP, STM32, ESP32, etc.) talking to the app:

1.  Connect your hardware to your computer using a USB cable or Serial adapter.

2.  Make sure your loaded config matches the bytes your device sends. For a framed protocol that usually means [Header] + [ID] + [Length] + [Data] + [CRC]; for Modbus RTU, set parser_type = modbus_rtu. See Section 5 for protocol details.

3.  Launch Bytehound. A bundled example config loads automatically; use Import Config if you want to load your own Excel workbook or CSV config folder.

4.  Click Connect in the toolbar. A Serial Connection Settings dialog will appear:

•  Select your hardware's COM port (e.g. COM7). Click Refresh if it isn't listed.

•  Set the Baud Rate to match your hardware (e.g., 115200).

•  Set Data bits, Stop bits, and Parity if needed (defaults suit most hardware).

•  Click Connect to open the port.

•  (Optional) Check Auto-reconnect on disconnect to have the app retry the connection automatically if the cable is pulled or the device resets.

Figure: Serial Connection Settings Dialog Interface

Once you click Connect:

•  The connection status LED (the circular ⬤ indicator on the left side of the bottom status bar) turns green.

•  The toolbar Connect button changes to Disconnect with a pink background.

Figure: Toolbar Disconnect Button and Status Bar LED Indicators when Connected

ℹ Note If the device is unplugged while monitoring, Bytehound automatically stops logging, closes the worker, and returns the Connect button to its ready state. If the port stays open but no data arrives for several seconds, the LED turns amber and the status reads Connected (No Data).

### Auto-Reconnect

When Auto-reconnect on disconnect is checked in the Connection dialog, the app monitors the serial worker for unexpected disconnects (USB pull, device reset, OS power management). On a drop it immediately schedules a reconnect attempt and uses exponential back-off so it doesn't hammer the USB stack:

| Attempt | Delay before retry |
| --- | --- |
| 1 | 1 s |
| 2 | 2 s |
| 3 | 4 s |
| 4 | 8 s |
| 5+ | 16 s (capped) |

Each attempt is logged to the Activity Log ([INFO] Auto-reconnect attempt N for COMx…). On success the app resumes normally, resets the counter, and logs [INFO] Auto-reconnect successful!. To cancel, click Disconnect or disable the checkbox before the next attempt.

ℹ Note Auto-reconnect does not re-start logging or Auto-Fetch automatically — it only re-opens the serial port. If you need logging to resume, start it manually after the reconnect notification appears in the Activity Log.

## Active Polling (Auto-Fetch)

If your hardware only sends data when asked (request/response protocol), connect first, then use the Start Auto-Fetch toolbar button. A dialog lets you choose which configured targets are active. The background worker then sends requests based on your config's polling_schedule sheet.

Figure: Start Auto-Fetch Polling Configuration Dialog

Pipelining and Timings: The Auto-Fetch dialog also allows you to configure Pipeline poll requests. By default, Bytehound sends one request and waits for its response (sequential). Pipelining allows multiple in-flight requests (controlled by Max in-flight), dramatically increasing throughput on hardware that supports it. You can also tune the TX gap (default 100 ms) which enforces a safe minimum spacing between consecutive outgoing polls.

When Auto-Fetch is active, the button text changes to Stop Auto-Fetch and turns pink. Click it again to stop. If your device streams data on its own, leave Auto-Fetch off.

## Unsolicited Data Detection

In many telemetry setups, a connected hardware device streams data continuously and automatically (unsolicited) without needing to be polled. If you attempt to enable Auto-Fetch (polling) on such a device, it will cause transmission collisions, serial line corruption, and packets will be dropped.

To protect the communication link, Bytehound includes automatic unsolicited data detection. If the application receives valid telemetry packets while Auto-Fetch is disabled, it flags the link as containing unsolicited streaming data. If you then click Start Auto-Fetch (or press F10), Bytehound will show a Potential Collision Warning dialog:

⚠ Warning Potential Collision Warning: The connected device is already streaming data automatically (unsolicited). Enabling Auto-Fetch (polling) may cause transmission collisions and corrupt the data. Do you want to enable Auto-Fetch anyway?

Figure: Potential Collision Warning Dialog

Clicking No aborts the Auto-Fetch request, ensuring the serial line is not saturated with conflicting commands. If your device supports both modes or has a specific hybrid query scheme, you can click Yes to override the warning and start polling anyway.

# 2. Core UI Panels & Features

The application is modular. The central Data Table stays in the middle, while dock panels such as Live Plot, Bitfields, Enums, Raw Console, and Activity Log can be dragged, stacked, resized, or hidden. Panel visibility is controlled under View → Panels, and View → Reset Window Layout restores the default arrangement.

Figure: Bytehound Main Application Window Layout Overview

## The Main Data Table

Located in the center of the screen, this read-only table displays all decoded variables in real time. The search box filters by text, the Group selector filters by configured signal group, and Show calculations controls whether calculated CalcGroups rows are visible. Columns include:

| Column | Description |
| --- | --- |
| Frame | Frame ID in hex (e.g.0x0010) |
| Group | Signal group from config, or the calculated group for CalcGroups rows |
| Variable | Signal name |
| Start B. | Start byte offset within the frame payload |
| Data Type | Decoded data type (e.g.uint16,float32) |
| Raw | Raw integer or float value before scaling |
| Value | Scaled engineering value, enum label, bitfield summary, or calculated value |
| Unit | Engineering unit (e.g. V, A, °C) |
| Status | Color-coded pill: OK, warnings, decode errors, enum labels, or active bitfield names |
| Updated | Timestamp of last received packet for this signal |

Right-click any row to add that signal to the Live Plot or copy its value to clipboard. Use Edit → Copy Table Snapshot (Ctrl+Shift+T) to copy the full table as tab-separated text — paste directly into Excel or Sheets for a quick bench snapshot without setting up full logging.

## Live Plot (Multi-Grid)

The Live Plot panel provides a fully flexible, multi-panel oscilloscope-style view of your telemetry data.

### Grid Layout

Use the Layout dropdown in the plot toolbar to select how many subplots to display:

| Layout | Panels | Best for |
| --- | --- | --- |
| 1×1 | 1 | Focus on a single signal group |
| 2×1 | 2 (stacked) | Compare two independent signal groups |
| 1×2 | 2 (side by side) | Wide-screen dual view |
| 1×3 | 3 (side by side) | Triple-wide comparisons |
| 3×1 | 3 (stacked) | Vertical trend comparison |
| 2×2 | 4 | Full dashboard view |
| 2×4 | 8 | High-density telemetry overview |
| 4×2 | 8 | Tall, multi-lane overview |

All panels share the same X-axis (time) — panning or zooming one panel moves all of them simultaneously.

Figure: Live Plot Panel in a Multi-Grid Layout

### Assigning Signals to Panels

Each panel has its own variable strip (now laid out using grid alignment for consistent stretching across multi-grid layout changes) above it:

•  Click + Add next to a panel label to pick a signal from your loaded config.

•  Each assigned signal appears as a coloured chip button (e.g. ● VoltageA  ✕). Click the chip to remove it.

•  Use the Y-scale dropdown on each panel to control how the Y-axis tracks data (see Y-axis Scale Modes below).

•  Alternatively, right-click any row in the Data Table → Add to Live Plot → choose the panel number from the sub-menu.

Panel assignments and layout are automatically saved and restored between sessions.

### Y-axis Scale Modes

Each panel strip has a small dropdown that picks how its Y-axis behaves on every redraw:

| Mode | Behaviour | When to use |
| --- | --- | --- |
| Y: Fit | Tight auto-fit with 5% padding. Axis rescales every 0.5 s to the visible data. | Default. Clean signals where you want the curve to fill the panel. |
| Y: Loose | Auto-fit with 25% headroom. Small noise stays inside the padding instead of triggering rescales. | Signals that wiggle a bit but you still want auto-tracking. |
| Y: Expand | Axis only grows, never shrinks. Once the bounds are large enough to contain the data, they stay put. | Noisy signals where Fit causes the axis to "breathe". Industry-standard noise killer (Vector CANalyzer'sAuto max, LabVIEW'sLoose Fit). |
| Y: Manual | Locked at a fixed range. Opens a dialog to type Y min / Y max; mouse pan/zoom still works and persists. | Signals with a known expected range (e.g. cell voltages 2.5 – 4.2 V). |

You can also right-click a panel → Set Y Range… to type bounds directly (this also switches the panel to Manual mode).

### View Modes

| Mode | Indicator | Behaviour |
| --- | --- | --- |
| 📊 Live | Default | X-axis always shows t=0 → current time. The view expands automatically as data accumulates. |
| 🔍 Explore | After any pan/zoom | View is frozen. New data still plots but the window doesn't move. Hint shows:▶ Live to resume. |

### Controls

•  ⏸ Pause / ▶ Live button (or Space): Toggles freeze/scroll. Clicking ▶ Live re-fits the Y axis and snaps X back to 0 → now.

•  Time Mode dropdown: Switch the X-axis labels between Elapsed (mm:ss since session start) and Clock (HH:MM:SS local wall-clock time). Your choice is saved between sessions.

•  Scroll wheel: Zoom in/out on time axis → enters Explore mode.

•  Click + drag: Pan the view → enters Explore mode.

### Trigger Configuration (Auto-Pause & Logging)

Click the Trigger... button on the Live Plot toolbar to define an automatic action based on a signal's value crossing a threshold. When the configured condition (e.g., Cell Voltage > 4.2) is met in incoming telemetry:

•  Pause Plot: Automatically freezes the plot (switches to Explore mode) so you don't lose the event off the left edge of the screen.

•  Start Logging: Automatically begins recording a Raw + Decoded telemetry session, capturing the event as it happens without manual intervention.

The trigger disarms automatically after firing once. A pink active state on the Trigger button indicates it is armed.

Figure: Live Plot Auto-Pause & Logging Trigger Settings Dialog

ℹ Note In Live mode the X-axis is anchored at 0 seconds (session start) and expands to the current elapsed time. Plot samples are kept in a bounded in-memory history (default 6,000 points per signal), so very long sessions may show older time ranges with sparse or no plotted points even though the axis still starts at zero.

## Bitfields Panel

If your hardware packs multiple Boolean flags into a single integer signal (like Fault states, Relay statuses, or Switch inputs), the Bitfields panel breaks them out into individual ON/OFF rows. The table columns are Frame, Variable, Bit, and State.

## Enums Panel

Sometimes hardware sends state integers (e.g. 0 = OFF, 1 = PRECHARGE, 2 = RUN). The Enums panel shows the latest mapping for each configured enum signal with Frame, Variable, Raw, and Label columns.

Figure: Bitfields and Enums Translation Dock Panels

## Raw Console

The Raw Console shows timestamped hexadecimal bytes flowing into and out of the application in real time. Lines are tagged as RX, TX, or ERR, and decode problems appear as [decode] or [decode warning] messages. Use it to debug communication errors or verify packet structures.

## Activity Log

The Activity Log tracks major application events: configurations loaded, connections established or lost, logging start/stop, and any error popups. Useful for post-mortem analysis of a test session.

Figure: Raw Bytes Console and Activity Log Dock Panels

Frame Errors: If the application receives corrupted telemetry packets (e.g., CRC mismatch, invalid payload length, or bad footer), the exact reason will be displayed here as a [WARN] Frame Error message (e.g. [WARN] Frame Error: CRC mismatch on frame 0x9000...). To prevent the log from being flooded during a noisy connection, identical error messages are throttled to appear at most once per second.

## Fault Alarms

Bytehound monitors all incoming decoded signals for the words fault or error (case-insensitive) in their Enum Labels or Bitfield states. If a fault is detected (e.g., a bit named "Overvoltage Fault" flips to ON), Bytehound automatically logs an [ALARM] entry to the Activity Log and the Raw Console. This requires no setup—as long as your Variables configuration includes Enums or Bitfields with those keywords, the application will catch them.

## Status Bar (bottom-right counters)

A single-line live readout sits in the bottom-right of the window with six fixed-width counters. They are updated continuously while connected and reset whenever you connect, load a new config, or click Edit → Clear Console / Log. The session clock (⏱) counts from the moment you connect — not from config load — so it always reflects actual device uptime. Reading the counters at a glance tells you whether the link is healthy and how the device is responding.

Frames: 1234 ¦ Errors: 0 ¦ Timeouts: 0 ¦ RX: 12.3 KB ¦ TX: 1.2 KB ¦ Lat: 9.8 ms

| Counter | What it means | What to watch for |
| --- | --- | --- |
| Frames | Total RX frames received and parsed since the session started — both valid and CRC-failed frames are counted here. | Should climb steadily while the device is talking. A flat counter means the device has gone silent. |
| Errors | Wire-level parse failures such as CRC mismatch (including Modbus CRC) or footer mismatch. The frame had the right header/length framing, but the integrity check failed. | Should stay at 0 on a healthy link. A slowly climbing number suggests electrical noise, a bad cable, or a baud-rate mismatch. A fast climb almost always means the protocol settings (CRC type, byte order, header) in your config don't match the device. |
| Timeouts | Poll requests that did not get a response within the configuredtimeout_ms. Primarily relevant whenAuto-Fetchis on; Modbus manual sends also wait for a response and can increment this counter. | 0 on a request/response device that is keeping up. A few timeouts on first connect are normal (the device may still be booting). A continuously rising count means the device is missing requests — try a longertimeout_msor a slowerinterval_ms. |
| RX | Total bytes received over the serial port since the session started, in B / KB / MB. | Roughly equal toFrames × frame size. A sudden plateau means the device stopped streaming. |
| TX | Total bytes transmitted by the app over the serial port — includes Auto-Fetch poll requests, TX-Command panel sends, and Parameter Editor writes. | 0 if you are only listening. Climbs each time you send a command or while polling is active. |
| Lat | Time between the two most recently received frames, in milliseconds. Lower = faster device. This is wall-clock latency between consecutive RX frames, not request-response latency. | For a 10 ms streamer expect ~10 ms here. A spike to hundreds of ms means the device hiccuped; if it stays high the link is choked. |

ℹ Note Errors vs decode errors. The Errors counter only tracks wire-level CRC failures. Configuration-level problems — for example a frame ID with no matching variables row, a payload shorter than the configured signal layout, or an unsupported data type — are reported in the Raw Console as [decode] … lines and are not counted here.

⚠ Warning ⚠️ Queue Saturated badge. When the serial packet queue or the raw-log flush queue fills up (e.g. very high baud rate, slow disk, or window drag stalling the GUI), a yellow ⚠️ Queue Saturated badge appears to the left of the counters. Incoming packets or log entries are being dropped to protect memory. Click the badge to dismiss it. Investigate the cause — reduce baud rate, increase flush interval, or free up system resources — then clear the badge and monitor whether it reappears.

Figure: QStatusBar Indicators, Telemetry Counters, and Queue Saturated badge

## Keyboard Shortcuts

All shortcuts are visible in the menu bar next to each action. Here is the complete reference:

| Shortcut | Action | Notes |
| --- | --- | --- |
| F9 | Connect / Disconnect | Toggles the serial connection. |
| F10 | Start / Stop Auto-Fetch | Opens the poll config dialog on first use. |
| Ctrl+L | Start / Stop Logging | Requires an active connection. |
| Ctrl+O | Import Config | Opens a file picker for.xlsxor CSV folder. |
| Ctrl+E | Export Template | Saves a blank config workbook. |
| Ctrl+K | Clear Console / Log | Resets all counters, clears decoded values and console. |
| Ctrl+Shift+C | Copy Value | Copies the selected table cell's value. |
| Ctrl+Shift+T | Copy Table Snapshot | Copies the full telemetry table as tab-separated text. |
| Ctrl+T | Analysis Suite | Opens the offline Analysis Suite window. |
| Ctrl+Q | Exit | Prompts if logging is active. |
| F1 | View Documentation | Opens this manual in your browser. |
| Space | Pause / Resume Live Plot | Toggles between Live and Explore modes. |

## The Menu Bar Reference

The main window's menu bar provides direct access to all configuration, control, view, and utility functions of the application. Here is an in-depth breakdown of every menu, action, and setting to help you operate the application effectively, even if you are new to serial testing tools:

### 1. File Menu

The File menu is where you manage Bytehound's connection mappings (decoders) and exit the application safely.

•  Import Config (Ctrl+O)What it does: Opens a file selection dialog to load a new protocol definition. You can select an Excel workbook (.xlsx or .xlsm) or a directory containing individual CSV sheets.
            Why you need it: Bytehound is generic; it doesn't know how your hardware packs its bytes until you tell it. Importing a configuration tells the app what each byte represents, what scales/offsets to apply, and how to format names.
            Example: You are testing a new Modbus-based temperature controller. You load modbus_temp_sensor_map.xlsx. Instantly, the main telemetry table updates to display rows for "Zone 1 Temp", "Heater Power %", and "Alarm Bitfield".

•  Export Template (Ctrl+E)What it does: Generates a fresh, blank Excel workbook containing the template structure (10 required sheets with correct headers).
            Why you need it: If you are building a new sensor or controller and want to construct a telemetry profile, this gives you a valid starting point without needing to remember column names.
            Example: You want to monitor a custom DIY microcontroller project. You click Export Template, save it as my_project_def.xlsx, and open it in Excel to start typing in your signal names, offsets, and data types (e.g. float32 at byte 4).

•  Exit (Ctrl+Q)What it does: Closes the main application.
            Safety Prompt: If the application is currently recording telemetry logs, exiting will show a caution popup. This warns you that closing the app will immediately stop logging and finalize the logs. This prompt is critical because closing the app abruptly without stopping the log would result in losing all buffered decoded data.

### 2. Edit Menu

The Edit menu allows you to copy real-time data quickly or clear diagnostic clutter from the display.

•  Copy Value (Ctrl+Shift+C)What it does: Copies the text contents of the currently selected cell in the center Data Table directly to the operating system clipboard.
            Example: You notice that the "Battery Voltage" signal wiggles to a peak value of 54.38 V and you want to document it. Highlight that cell in the table, press Ctrl+Shift+C, and paste the value into Slack, Notepad, or your bug tracking tool.

•  Copy Table Snapshot (Ctrl+Shift+T)What it does: Captures the complete, real-time status of all decoded signals (columns and rows) in the main table and copies them as tab-separated values (TSV) to the clipboard.
            Example: You are running a bench test and want to quickly record the current status of all 50 cell voltages without starting a full session log. Press Ctrl+Shift+T, open Microsoft Excel or Google Sheets, click on cell A1, and press Paste. The entire table will populate instantly with aligned columns.

•  Clear Console / Log (Ctrl+K)What it does: Resets all communication statistics (Frames, Errors, Timeouts, RX, TX, Latency) back to zero, clears all historical lines from the Raw Console and Activity Log text panes, and empties the main table's variables.
            Why you need it: If you have resolved a wiring issue that caused thousands of CRC errors, you want to clear the error count so you can monitor if any *new* errors occur. Pressing Ctrl+K gives you a clean slate.

### 3. View Menu

The View menu manages visual layout docking, themes, and configuration metadata.

•  Panels (Submenu)What it does: Contains checkable options for each of the seven interface docks: *Live Plot*, *Bitfields*, *Enums*, *TX Commands*, *Parameter Editor*, *Raw Console*, and *Activity Log*. Unchecking a panel closes it; checking it opens it.
            Why you need it: Docks can be dragged, floating independently, or closed. If you accidentally close a panel (like the Live Plot), you can restore it here.
            Example: When doing read-only telemetry checks, you don't need to send commands. You uncheck *TX Commands* and *Parameter Editor* to hide them, freeing up screen space for the plot panel.

•  Reset Window Layout (Ctrl+Shift+R)What it does: Restores all dock panels to their default positions and splits, and returns the window size to standard defaults.
            Example: You dragged the Live Plot to a second monitor and closed the console, and now your layout is disorganized. Pressing Ctrl+Shift+R snaps all panels back to their neat, default locations.

•  Config Info...What it does: Displays a dialog summarizing the active configuration profile, showing the total number of parsed variables, valid frame IDs, CRC settings, and the serialdefaults configuration. Below the summary, the Frame Format Diagram renders a graphical, byte-aligned layout of the on-wire packet structure.
            Tabs and Options:Hover over any field block in either tab to see its name, byte offset, and size in bytes.

•  RX Frames / Registers: Shows the byte layout of incoming frames or Modbus read registers. Use the dropdown to inspect each frame's variables.

•  TX Commands: Shows the byte layout of outgoing command packets, including static payload bytes, command fields, and any automatic packet padding. Use the dropdown to choose any configured command.

•  Theme (Submenu)What it does: Switches the visual appearance of the application.

•  Dark: Best for low-light test benches or server rooms, reducing eye strain.

•  Light: Best for bright outdoor environments, test labs, or creating clean, high-contrast screenshots for technical documentation.

•  System: Automatically aligns the app with your Windows operating system light/dark theme settings.

### 4. Device Menu

The Device menu controls the physical hardware communication and data recording sessions.

•  Connect / Disconnect (F9)What it does: Opens the Serial Settings dialog (allowing you to select COM Port, Baud rate, data/stop bits, parity, and timeout) and connects the serial interface. Toggles the connection on/off.
            Example: Plug in your USB-to-Serial adapter. Press F9, click Refresh, select COM5, set the baud rate to match your hardware (e.g. 115200), and click Connect. The connection LED will turn green.

•  Start / Stop Auto-Fetch (F10)What it does: Toggles active register polling on devices that follow a request/response (polled) protocol rather than streaming data continuously.
            Example: Your Modbus sensor requires a "Read Input Registers" command to be sent before it replies with the temperature. Enabling Auto-Fetch starts sending these commands automatically at intervals defined in the polling_schedule sheet.

•  Start / Stop Logging (Ctrl+L)What it does: Opens the session recording dialog and begins logging data.
            Modes:

•  Raw only: Records raw incoming and outgoing hexadecimal bytes with timestamps directly to a *_raw.csv file. Safest for deep diagnostics.

•  Decoded only: Buffers decoded variables into memory and formats them as a wide Excel spreadsheet (*_decoded.xlsx) with metadata, written on Stop.

•  Raw + Decoded: Records both files simultaneously (highly recommended).

### 5. Tools Menu

The Tools menu hosts the post-test visual analysis suite and utility preferences.

•  Analysis Suite (Ctrl+T)What it does: Opens a separate, non-modal plotting window for offline log analysis.
            Why you need it: It lets you load saved *_decoded.xlsx files from prior test runs, zoom in, place measurement cursors, apply smoothing filters, and overlay multiple runs to compare hardware behavior.

•  Logging Settings...What it does: Opens the configuration dialog for internal rotating diagnostics and disk writing schedules.
            Understanding the Settings:

•  Log level (DEBUG / INFO / WARNING / ERROR): Controls how much diagnostic information the application writes to its internal bytehound.log file. If you set it to INFO, it only records major events (starts, connections). If you set it to DEBUG, it logs every transaction details, worker threads, and layout swaps—excellent for debugging connection drops or serial driver glitches.

•  Flush interval (seconds): Controls how frequently the Raw CSV logger writes buffered data from RAM to your disk. The default is 0.5 s. Setting it to 0.0 s forces an immediate disk write on every packet. This protects against data loss if the computer suddenly loses power, but it causes continuous disk activity which can degrade performance on older or slower drives.

•  Plot Settings...What it does: Opens settings for the Live Plot panel.
            Understanding the Settings:

•  Cap live plot memory history — checkbox that enables a hard limit on how many samples the Live Plot stores per signal. When checked, set the Memory cap limit spinbox to the desired maximum (1 000 – 10 000 000 samples). When unchecked the history grows unbounded for the lifetime of the session; on long or high-rate sessions this can consume significant RAM.

•  Default display window — controls how wide the Live Plot X-axis is when first switched to Live mode. Options: 30 seconds, 1 minute, 2 minutes, 5 minutes (default), 10 minutes, 30 minutes, or All session. Choosing All session always shows every sample from t=0; on long sessions this can make individual events hard to see — switch to a shorter window for real-time monitoring.

### 6. Help Menu

The Help menu provides manual documentation, software update tools, and debugging diagnostics.

•  View Documentation (F1)What it does: Opens this manual (index.html) in your default web browser for quick offline reference.

•  Check for UpdatesWhat it does: Safely connects to the online updates server to see if a new release is available.
            Why it is safe: It displays the version numbers and release notes first. If you confirm, it downloads the installer, computes its **SHA-256 cryptographic checksum**, and compares it with the server manifest. If the checksum matches, it runs the installer in silent mode to update your installation automatically. If it doesn't match, it blocks the install to protect you from corrupted downloads.

•  Copy DiagnosticsWhat it does: Automatically gathers details about your Windows version, Python version, PySide6/Qt versions, serial port configuration, data counters (frames, errors), and the last 200 lines of the application log. It copies this info to your clipboard.
            Why you need it: If you run into a bug, you can click this action and immediately paste the results into a bug report or support email, giving developers all the context they need to help you in one click.

•  Report Issue...What it does: Opens a dialog to report software issues directly from within the application.
            How it works: Enter a title and description, and optionally check the option to include the tail of the application diagnostics log (last 200 lines of bytehound.log). When you submit, the application securely forwards your description along with complete **system and session diagnostics** (including Windows/OS version, Python/Qt/PySide6 versions, frozen build status, active configuration path, connection status, serial port settings, and real-time telemetry counters such as frames received, CRC errors, timeouts, and I/O byte counts).
            What you should include manually: To help developers reproduce and rectify bugs quickly, please describe the exact steps to trigger the bug, the expected behavior vs. actual behavior, and consider sharing your custom Excel/CSV configuration workbook if the issue is config-dependent.

•  About BytehoundWhat it does: Opens a window showing copyright notices, version info, credits, and links to the MIT License.

# 3. Controlling Your Hardware

The app isn't just for reading data — you can actively command your hardware over the same serial connection.

## TX Commands

The TX Commands panel provides config-defined commands for sending predefined packets to your hardware. Choose a command from the selector, review the generated Packet Preview, then click Send. This is useful for triggering resets, clearing faults, or toggling modes.

Commands with input fields (defined in tx_command_fields) show an inline numeric form. The app validates ranges, applies scale/offset in reverse, appends the encoded field bytes after any static payload_hex, and builds the full packet automatically. Click Build to refresh the preview without transmitting.

ℹ Note Send requires a connection. Packet preview works from the loaded config even before a serial port is open, but Send will refuse to transmit until you connect.

Figure: TX Commands Panel User Interface with Parameter Inputs and Hex Packet Preview

## Parameter Editor

The Parameter Editor panel shows all signals in your config that are marked as writable (read_write = RW or W). For each signal you can:

•  See the Live Value column update in real-time as packets arrive.

•  Type a new value (in engineering units, e.g. 12.5 for 12.5 V) in the Write column's input field. Hover over the input to see the numeric range inferred from min_value, max_value, and the signal type.

•  Press Enter or click the Write button. The app validates the range, reverses the scale/offset, encodes the raw bytes, appends the correct checksum, and transmits the packet immediately as a priority TX.

Figure: Parameter Editor Panel with Live Values and Editable Fields

### How the value is encoded

The encoding formula is the exact inverse of how the signal is decoded:

raw = (your_value - offset) / scale encoded_bytes = raw → data_type (uint8/16/32, int8/16/32, float32/64) → byte_order

| Protocol | What gets sent |
| --- | --- |
| Framed | A complete framed packet is built for the signal'sframe_id. The encoded bytes are placed at the exactstart_byteoffset (zero-padded payload). Header, length, CRC, and footer are all added automatically per the Protocol sheet. |
| Modbus RTU | An FC 06 (single-register) request is built when the encoded payload is 2 bytes; FC 16 (multi-register) is used for longer payloads. The raw value is encoded using the signal'sscale,offset, and byte order, and CRC16/Modbus is appended automatically. |

⚠ Warning Writes require an active serial connection. If no signals appear in the editor, all signals in your config are set to read_write = R (read-only). Update your Excel config to mark the appropriate signals as RW or W.

ℹ Note Parameter Editor writes are recorded in the Activity Log with the signal name, engineering value, unit, and full raw hex. TX Command sends are also echoed in the Raw Console as TX lines.

## Reading a TX packet (byte-by-byte)

Every TX line you see in the Raw Console — whether it came from a TX Command, the Parameter Editor, or an Auto-Fetch poll request — follows the same wire format defined by your Protocol sheet. Use this guide to decode any TX line.

### Framed packet — variable length (no tx_pad_length)

Reference packet from the bundled template (Set_Voltage_Limit at 58.5 V on frame 0x0120):

TX, AA 55 01 20 02 49 02 3C 9E EE

AA 55 ← header (2 bytes, from Protocol.header) 01 20 ← frame_id 0x0120 (big-endian, from frame_id_size + byte_order) 02 ← length = 2 (payload bytes only, from length_meaning=payload_only) 49 02 ← payload, little-endian uint16 = 0x0249 = 585 → 585 × 0.1 = 58.5 V 3C 9E ← CRC16 over header..payload (crc_type, crc_size, crc_byte_order) EE ← footer (1 byte, from Protocol.footer)

### Framed packet — fixed length (tx_pad_length set)

Same protocol, but with tx_pad_length = 16 and a write of 123 to a uint16 signal on frame 0x0050:

TX, AA 55 00 50 02 7B 00 00 00 00 00 00 00 00 00 00

AA 55 ← header (2 bytes) 00 50 ← frame_id 0x0050 02 ← length = 2 (payload only — padding is not counted) 7B 00 ← payload, little-endian uint16 = 0x007B = 123 00 00 00 00 00 00 00 00 00 ← 9 bytes of zero-padding to reach tx_pad_length=16

⚠ Warning If your packet ends in zeros with no CRC or footer visible like the example above, that means your config has crc_type = none and an empty footer. If you expected a CRC and footer, open your Protocol sheet and check those two fields. With a CRC and footer configured, the last bytes of the frame would be the CRC + footer, and the zero-padding would sit between the payload and the CRC. tx_pad_length is the total frame length on the wire, including CRC and footer.

### Modbus RTU — single-register write (FC 06)

With parser_type = modbus_rtu and modbus_node_address = 1, writing 123 to register address 0x0050:

TX, 01 06 00 50 00 7B 99 8C

01 ← node address (from Protocol.modbus_node_address) 06 ← function code: 06 = Write Single Register 00 50 ← target register address 0x0050 (big-endian, per Modbus spec) 00 7B ← value to write, big-endian uint16 = 0x007B = 123 99 8C ← CRC16/Modbus over preceding bytes, little-endian (per Modbus spec)

ℹ Note Modbus byte order is fixed by the spec: addresses and values are big-endian, the CRC is little-endian. The Protocol sheet's byte_order fields are ignored for Modbus.

### Quick reference — where each byte comes from

| Byte(s) | Source | How to change it |
| --- | --- | --- |
| Header | Protocol.header | Edit the Protocol sheet'sheaderhex value. |
| Frame ID | Variable'sid_or_address(or TxCommands row) | Set in Variables / TxCommands. |
| Frame ID size + byte order | frame_id_size,frame_id_byte_order | Protocol sheet. |
| Length | Computed from payload size | Controlled bylength_size,length_byte_order,length_meaning. |
| Payload bytes | Encoded from your value viaraw = (value − offset) / scale | Setscale,offset,data_type,byte_orderon the variable. |
| Zero-padding (if any) | Protocol.tx_pad_length | Set or cleartx_pad_length. Padding sits between payload and CRC. |
| CRC | Computed overcrc_coverageregion | crc_type,crc_size,crc_byte_order,crc_coverage. |
| Footer | Protocol.footer | Edit the Protocol sheet'sfooterhex value. |

# 4. Logging and Data Analysis

## Recording Logs

To save your telemetry session for later review:

1.  Ensure you are connected and receiving data.

2.  Click Device → Start Logging (or use the toolbar). Choose Raw + Decoded, Raw only, or Decoded only, then pick the base filename. The button turns pink while logging is active.

3.  The default folder is ~/Documents/Bytehound/. Depending on the selected mode, the app writes:

•  *_raw.csv — timestamped RX/TX hex frames with timestamp, direction, hex, and delta_t_ms columns. The byte-level capture of the session.

•  *_decoded.xlsx — Excel workbook with two sheets: Metadata (app, version, port, baud, config path, session start, logging mode, etc.) and Data (one wide row per complete poll cycle, with a per-frame block of elapsed_ms + frame_id + signals).

4.  When finished, click Device → Stop Logging. This finalises the decoded workbook.

Figure: Start Logging Session Configuration Dialog

ℹ Note The raw CSV is flushed periodically (default 0.5 s, configurable under Tools → Logging Settings) and once more on close. The decoded .xlsx is buffered in memory and saved on Stop Logging — if the app crashes before Stop, the decoded workbook is lost. A session config snapshot is saved beside the log so the same decoder can be re-applied later.

## Logging Settings

Under Tools → Logging Settings, you can configure application-level diagnostic logging and data flushing:

•  Log level: Sets the verbosity of Bytehound's internal rotating log file (bytehound.log). INFO is the default; use DEBUG if you are troubleshooting parse failures or protocol issues.

•  Flush interval: Controls how frequently the raw *_raw.csv telemetry log writes to disk. The default is 0.5 s. Set to 0.0 to flush on every write, which ensures no raw data is lost if the app crashes, though it may cause a slight performance penalty on slower disks. (Decoded .xlsx logs are always kept in memory and only written on Stop.)

## The Analysis Suite

For deep-diving into saved test data and comparing multiple runs. The window is non-modal, so you can keep it open while the main app is running.

Figure: Offline Log Analysis Suite Interface

### Open and load logs

1.  Go to Tools -> Analysis Suite.

2.  Click Load Logs (or File -> Load Logs...) and choose one or more files.

3.  Supported formats: Bytehound decoded .xlsx, legacy decoded .csv, and external Dyno-style .xlsx exports.

•  Visibility: use the checkbox beside each log to show or hide it.

•  Color: click the colored dot to change the log color.

•  Offset: use the time offset spin box (seconds) to align runs.

•  Remove: the Remove button drops the most recently loaded log.

### Layout and parameters

Use the Layout dropdown to choose the subplot grid. All subplots share a linked X-axis.

•  Check/clear: the checkbox beside each parameter toggles its curve.

•  Search: filter parameters with the search box; the hit counter shows matches.

•  Group/ungroup: multi-select parameters, then use Group / Ungroup to control overlays.

•  Drag and drop: drag parameters between subplots, or create a new empty subplot and drop into it.

•  Right-click: move parameters to another subplot, normalize curves to 0-1, enable smoothing, or delete a subplot.

### Plot behavior and view tools

•  Hover crosshair: the status bar shows time and values under the cursor.

•  Multi-param encoding: in multi-parameter subplots, color indicates parameter and line style indicates log; in single-parameter plots, color tracks the log.

•  Auto-fit Y:View -> Auto-Fit Y to Visible Range re-scales Y after X zoom/pan (F).

•  Wall clock:View -> Wall Clock Time (X-Axis) swaps elapsed time for HH:MM:SS when the log has a start timestamp.

•  Reset zoom:View -> Reset Zoom or Ctrl+0.

### Cursors and statistics

•  Vertical cursors:V adds a cursor on the hovered subplot; Shift+V adds one across all subplots.

•  Horizontal cursor:H adds a Y cursor at the current mouse position.

•  Edit: drag cursors to measure deltas; Delete removes the selected cursor; Tools -> Clear All Cursors wipes them.

•  Cursors tab: now shows a clean table with Cursor, Log, Parameter, Value, Unit, and Delta.

•  Vertical deltas: when two vertical cursors are active, the Delta column shows C2 minus C1 for each curve.

•  Horizontal deltas: when two horizontal cursors are on the same subplot, the Delta column shows H2 minus H1.

•  Cursor labels: horizontal cursors are labeled in-plot as H1: value, H2: value and also appear as H1 @ value in the table.

•  Statistics tab: shows min, P5, mean, median, P95, max, std, and n for the visible X-range of each curve.

ℹ Note Statistics glossary:P5 / P95: 5th and 95th percentiles (values below/above which 5% of samples fall).Std: standard deviation, a measure of spread around the mean.n: number of samples in the current visible X-range.

ℹ Note Example cursor table:CursorLogParameterValueUnitDeltaC1 @ 1:14serial_logPack Voltage53.81VC2 @ 2:16serial_logPack Voltage48.73V-5.08H1 @ 48.7serial_logPack Voltage48.70VH2 @ 49.2serial_logPack Voltage49.20V0.50

### Export and sessions

•  Sessions:File -> Save Session stores layout, checked params, cursors, and view range; Load Session... restores them.

•  CSV export:File -> Export Visible Data as CSV... writes a wide table over the current X-range. If the exported range is very dense (e.g. over 100,000 points), Bytehound will automatically prompt you to apply a decimation factor to keep the CSV file small and Excel-friendly.

•  Images/PDF:File -> Export Plots as Image... or Export Plots as PDF....

•  Per-plot export: right-click a subplot to export its visible range or copy the plot image.

•  Folders:File -> Open Data Logs Folder and Open Analysis Folder jump to the saved output locations.

### Menu reference (quick guide)

Short summary of where common actions live:

•  File: load logs, manage sessions, export CSV/images/PDF, open output folders, close window.

•  View: auto-fit Y, reset zoom, switch Cursors/Statistics tab, set smoothing window, toggle wall-clock time.

•  Layout: group/ungroup parameters, add a new empty subplot.

•  Tools: add or clear vertical/horizontal cursors.

•  Tools → Import Schema Mapper...: configure how the Analysis Suite reads external log files that don't follow Bytehound's default decoded .xlsx format. Three settings are available:
            
            Settings are persisted between sessions. Click Save to apply; changes take effect the next time you load a log file.

•  Sheet Names — comma-separated list of worksheet names to try when opening a workbook (e.g. Data,Record,Sheet1). The first matching sheet is used.

•  Time Columns — comma-separated list of column headers that represent elapsed time (e.g. Elapsed (s),elapsed_ms). The first matching column is used as the X-axis.

•  Time Scale Mapping — one column_name: scale_factor pair per line to convert the raw value to seconds (e.g. elapsed_ms: 0.001 converts milliseconds to seconds). Leave blank for columns already in seconds.

Figure: Import Schema Mapper Dialog

Additional Analysis tools include:

•  Scatter: open the X-Y Plotter. The Plotter allows you to plot any two signals against each other (e.g., Voltage vs Current) across the visible time range, automatically applying 1D linear regression (best-fit line) and calculating the R-squared correlation coefficient. The X-Y Plotter window fully respects the active Light/Dark/System theme for consistent visual appearance.

Figure: Scatter X-Y Plotter Window with Linear Regression

ℹ Note Quick tips:Hover any subplot to see live values in the status bar.Use V / Shift+V for vertical cursors and H for a horizontal cursor.Press Delete to remove the selected cursor.Right-click a subplot to export its visible range or copy an image.Right-click a subplot row in the parameter tree to normalize or smooth that subplot.Use the search box to filter parameters without losing your layout.

✔ Tip Common workflows:Align two runs: load both logs, then adjust the time offset spin box on one log until key events line up.Compare signal deltas: add two vertical cursors and read the X and Y values in the Cursors tab.Normalize mixed units: group the parameters, then right-click the subplot header and enable Normalize (0-1).Clean up noisy data: enable smoothing on a subplot and set the window size under View -> Smoothing Window...Export a report snapshot: zoom to a range, then export CSV or PDF from the File menu.

# 5. Customizing Your Data (The Excel Config)

The Bytehound uses an Excel workbook to know how to decode your hardware's raw bytes, what to call each signal, how to scale it, and how to send commands back. You do not write any Python code — just fill in the sheets.

ℹ Note Quick start: Click File → Export Template, save the .xlsx file, open it in Excel, fill in the sheets described below, save, then click File → Import Config to load it. Changes take effect immediately.

ℹ Note Alternative CSV Folder Configuration: Instead of a single Excel .xlsx workbook, you can configure Bytehound using a directory containing individual CSV files. When importing, simply select any CSV file inside the configuration folder, and Bytehound will automatically read all related CSV files in that directory. The CSV filenames are case-insensitive and support normalization aliases: protocol.csv (maps to Sheet 1 — Protocol)frames.csv (maps to Sheet 2 — Frames)variables.csv or frame_variables.csv or framevariables.csv (maps to Sheet 3 — Variables)bitfields.csv (maps to Sheet 4 — Bitfields)enums.csv (maps to Sheet 5 — Enums)calc_groups.csv or calcgroups.csv (maps to Sheet 6 — CalcGroups)tx_commands.csv or txcommands.csv (maps to Sheet 7 — TxCommands)tx_command_fields.csv or txcommandfields.csv (maps to Sheet 8 — TxCommandFields)polling_schedule.csv or pollingschedule.csv (maps to Sheet 9 — PollingSchedule)serial_defaults.csv or serialdefaults.csv (maps to Sheet 10 — SerialDefaults)frame_config.csv or frameconfig.csv (maps to Legacy — FrameConfig) Each CSV file must contain a header row matching the exact column names documented for the corresponding sheet below.

### Config Validator CLI

Bytehound can be run headlessly from the command line to validate configuration syntax without launching the GUI. Simply execute:

Bytehound.exe --validate config.xlsx

If the configuration schema is perfect (valid byte orders, correct types, no missing required fields), the CLI will print a success summary and exit with code 0. If there are schema errors, it prints the errors and exits with code 1, failing the build pipeline instantly.

The workbook contains the following sheets in this order. Each section below matches one sheet tab.

## Sheet 1 — Protocol

Defines the wire-level packet structure. Exactly one row must have enabled = TRUE. Do not edit this sheet unless you designed the packet format.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| profile_name | text | BMS_v2 | Free-text label for this protocol profile. |
| parser_type | enum | framed | framed= custom byte-framed protocol (default for most embedded MCUs).modbus_rtu= standard Modbus RTU — framing, FC codes, and CRC are handled automatically. |
| modbus_node_address | integer | 1(default) | Modbus slave address used as the first byte of every outgoing Modbus request. Only meaningful whenparser_type = modbus_rtu; ignored forframed. Defaults to1if blank or omitted. |
| header_hex | hex bytes | AA 55 | Fixed start-of-frame marker the parser searches for. Space-separated hex bytes.Ignored when parser_type = modbus_rtu. |
| frame_id_size | integer | 2 | How many bytes carry the frame ID after the header.Ignored for modbus_rtu. |
| frame_id_byte_order | enum | little | bigorlittleendian for the frame ID bytes. |
| length_size | integer | 1 | How many bytes carry the payload-length field.Ignored for modbus_rtu (length is implicit in each FC). |
| length_meaning | enum | payload_only(default) | What the length field counts on the wire. Four values supported:payload_only(just the payload bytes),frame_total(header + frame_id + length + payload + CRC + footer — the full frame size),header_to_crc(everything except the footer),payload_plus_crc(payload + CRC bytes). The parser and TX builder honour the chosen value end-to-end.Ignored for modbus_rtu. |
| crc_type | enum | crc16_modbus | Checksum algorithm:crc16_modbus,crc16_ccitt,crc32, ornone. For modbus_rtu the CRC is always CRC16/Modbus regardless of this field. |
| crc_size | integer | 2 | How many bytes the CRC occupies at the end of the frame.Ignored for modbus_rtu. |
| crc_byte_order | enum | little | bigorlittlebyte order for the CRC field.Ignored for modbus_rtu. |
| footer_hex | hex bytes | (leave blank) | Optional end-of-frame marker. Leave blank if your protocol has no footer.Ignored for modbus_rtu. |
| raw_log_format | enum | hex(default) | Controls how the raw CSV logger formats thehexcolumn.hex(default) writes space-separated uppercase bytes likeAA 55 01 20.compactwrites contiguous bytes likeAA550120— smaller files, faster to grep. Invalid values are rejected at load time. |
| inter_frame_delay_ms | integer | 10 | Minimum gap (ms) between consecutive TX requests. Default 10 ms. |
| tx_pad_length | integer | 16(or blank) | Forces every outgoing TX packet to be exactly this many bytes on the wire. Short commands are zero-padded between payload and CRC; CRC is computedoverthe padding so the receiver's check still passes. Leave blank to send variable-length packets.Framed only — see Fixed-length TX below. |
| length_byte_order | enum(blank / big / little) | (leave blank) | Byte order for the length field. Leave blank to inheritframe_id_byte_order— that covers almost every protocol. Only set this explicitly if your protocol uses multi-byte lengths in a different endianness from the frame ID. |
| crc_coverage | enum | header_to_payload(default when blank) | Which bytes the CRC is computed over. Four values supported:header_to_payload(header + frame_id + length + payload — the usual choice),frame_id_to_payload(skips the header),payload_only(just the payload),full_frame(everything except the CRC field itself, i.e. header + frame_id + length + payload + footer). The parser and TX builder use the same coverage so build/parse round-trips are byte-exact. |
| escape_mode | enum | none(default when blank) | Outer byte-stuffing / delimiter scheme appliedaroundeach Bytehound frame on the wire. Four values supported:none(current behaviour — frames are delimited by theheadersignature with no escaping),slip(RFC 1055: 0xC0 delimiter, 0xDB escape),hdlc(HDLC byte-stuffing: 0x7E flag, 0x7D escape with XOR 0x20),cobs(Consistent Overhead Byte Stuffing: 0x00 delimiter, code-prefixed chunks). The parser's outer un-framer runs first; the existing header / length / CRC / footer logic then processes the un-escaped inner frame. |
| enabled | TRUE/FALSE | TRUE | Only one row may beTRUE. The app uses that row and ignores all others. |

ℹ Note length_meaning, crc_coverage, and escape_mode all support multiple values now. Leave them blank to inherit the most common defaults (payload_only, header_to_payload, none) — these are what 99% of custom embedded protocols use.

⚠ Warning Interop caveat for the length_meaning and crc_coverage alternates. Bytehound's parser and TX builder agree end-to-end on all 16 combinations (32 build → parse round-trip tests cover them with and without a footer). What that proves is internal consistency: Bytehound can decode what Bytehound builds. It does not prove that Bytehound's interpretation of frame_total / header_to_crc / payload_plus_crc / frame_id_to_payload / payload_only / full_frame matches another vendor's protocol that happens to use the same name. Concretely: frame_total here counts every byte from the start of the header through the footer inclusive — header + frame_id + length + payload + CRC + footer.header_to_crc counts header + frame_id + length + payload + CRC (footer excluded).payload_plus_crc counts payload + CRC only.full_frame CRC covers header + frame_id + length + payload + footer (every byte except the CRC field itself).frame_id_to_payload CRC covers frame_id + length + payload (header excluded from CRC).payload_only (CRC) covers just the payload bytes. If you connect Bytehound to a real device that uses one of these names with even slightly different boundaries, the build/parse will silently mismatch the device — every TX frame will fail the device's CRC, every RX frame will fail Bytehound's CRC. Test the first byte sequence on the wire against your device's reference implementation before trusting these alternates in production.

⚠ Warning Same caveat for the escape_mode alternates. Bytehound implements SLIP per RFC 1055, HDLC byte-stuffing with the standard 0x7E flag / 0x7D escape / XOR 0x20 mapping, and COBS per Cheshire & Baker (1999). 134 round-trip tests cover every combination of escape_mode × length_meaning × crc_coverage × footer, plus targeted tests for the worst-case payloads (all-special bytes, frames chunked one byte at a time, three frames in one feed). Internal consistency is proven; vendor interop is not. Some HDLC implementations escape additional control bytes (0x11 / 0x13 XON/XOFF), some COBS variants insert a phantom zero, and at least one well-known "SLIP" device drops the leading 0xC0. Always verify the first wire bytes against your device's reference output before deploying.

✔ Tip Wire format (framed):[ header ] [ frame_id ] [ length ] [ payload ] [ CRC ] [ footer? ]Wire format (modbus_rtu):[ node_addr(1) ] [ FC(1) ] [ data bytes ] [ CRC16(2) ] — FC03/FC04 read, FC06/FC16 write

ℹ Note modbus_rtu is fully supported. Set parser_type = modbus_rtu and the parser and TX builder switch to standard Modbus RTU mode automatically. In this mode the id_or_address column in Variables and PollingSchedule is the Modbus register address (not a frame ID).

### Fixed-length TX (tx_pad_length) — for buffer-full RX interrupts

Many microcontrollers receive serial data using a buffer-full or DMA-complete interrupt: the ISR fires once N bytes have arrived, and the firmware parses that fixed-size block. If your host sends commands of varying length, those firmwares either miss bytes, split commands across two interrupts, or sit waiting for bytes that never come. Setting tx_pad_length makes Bytehound transmit a constant-size frame for every command, regardless of payload.

Wire layout when padding is active:

[ header ][ frame_id ][ length ][ payload ][ 0x00 0x00 … ][ CRC ][ footer? ] └──────────────── padded region ────────────────┘ └──────────────── tx_pad_length total bytes ────────────────────────────────┘

•  Padding bytes (0x00) are inserted between the payload and the CRC.

•  The CRC is computed over header + frame_id + length + payload + padding, so the receiver's checksum still matches without any firmware change.

•  The length field still reports the real payload length (not including padding). Firmware that uses buffer-full RX usually ignores this field anyway — it knows the frame size from the buffer size.

•  Only TX is affected. Received frames are still parsed by header + length + CRC as normal.

How to pick the value: set tx_pad_length equal to your MCU's RX buffer size (or DMA transfer count). Example: an STM32 with HAL_UART_Receive_IT(&huart1, buf, 16) needs tx_pad_length = 16.

✔ Tip Worked example. Protocol with header = AA 55, frame_id_size = 2, length_size = 1, crc_size = 2, no footer, and tx_pad_length = 16: Command with 1-byte payload 0x42 for frame 0x0010 goes out as:AA 55 10 00 01 42 00 00 00 00 00 00 00 CRC_lo CRC_hi Total = 2 + 2 + 1 + 1 + 7 + 2 = 16 bytes. The CRC covers the first 14 bytes, padding included.

⚠ Warning If a command is bigger than tx_pad_length, the app errors out instead of silently sending an oversized frame — you'll see TX frame (id=0x…) is N bytes but tx_pad_length is M. Either raise tx_pad_length or shrink the command. Also, tx_pad_length must be at least crc_size + len(footer), otherwise there's no room for the CRC and the app rejects the config.

ℹ Note Not for Modbus RTU. Modbus has its own fixed-per-FC framing — tx_pad_length is ignored when parser_type = modbus_rtu.

Bundled template content (the single enabled row that File → Export Template writes — blanks for length_byte_order, crc_coverage, escape_mode, and tx_pad_length mean "use the sensible default"):

profile_name = Default parser_type = framed header_hex = AA 55 frame_id_size = 2 frame_id_byte_order = little length_size = 1 length_meaning = payload_only length_byte_order = ← inherits frame_id_byte_order crc_type = crc16_modbus crc_size = 2 crc_byte_order = little crc_coverage = ← defaults to header_to_payload footer_hex = EE ← matches the expected frame trailer byte escape_mode = ← defaults to none raw_log_format = hex ← `hex` = spaced, `compact` = contiguous inter_frame_delay_ms = 10 tx_pad_length = ← variable-length TX modbus_node_address = 1 ← Modbus slave address (ignored when parser_type = framed) enabled = TRUE

## Sheet 2 — Frames

Optional. Gives each frame ID a human-readable name and a per-frame expected payload length. If you omit this sheet, frame names are derived automatically from the Variables sheet.

ℹ Note payload_length is per-frame, not global. Each frame row declares its own expected payload size — one frame can carry 8 bytes while another carries 28 bytes. Leave payload_length blank for frames where you do not want length validation. Do not put a payload length in the Protocol sheet; that sheet only describes the wire framing (header, CRC, etc.).

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| frame_id | hex(required) | 0x0010 | Frame identifier as sent by hardware. Can be0x10,10, or16(decimal). |
| frame_name | text(required) | BMS Status | Display name shown in the Data Table's Frame column. |
| payload_length | integer | 8(blank = no validation) | Expected payload size in bytesfor this frame only. If the received payload differs, a decode warning appears in the Raw Console and Activity Log. Leave blank to skip validation for this frame. |
| direction | enum | rxtx(default) | Whether the frame can be received, transmitted, or both —controls UI gating.rx: telemetry-only — frame is parsed when received, but hidden fromTX Commandsand from theParameter Editor(signals on rx-only frames cannot be written).tx: command-only — never expected on the RX wire.rxtx(default when blank): both directions. Case-insensitive. Invalid values are rejected at load time. Auto-created frames default torxtx. |
| enabled | TRUE/FALSE | TRUE(default) | SetFALSEto completely ignore this frame — the row is skipped at load time and no signals from it are decoded. |
| description | text | Battery pack telemetry(blank ok) | Optional free-text note — not displayed in the app. |

Bundled template content (what File → Export Template produces — same 3 frames the bundled Variables sheet references):

frame_id ¦ frame_name ¦ payload_length ¦ direction ¦ enabled ¦ description 0x1000 ¦ BMS Status ¦ 8 ¦ rxtx ¦ TRUE ¦ Telemetry + Reset Faults TX target 0x2000 ¦ BMS Settings ¦ 8 ¦ rxtx ¦ TRUE ¦ Telemetry + Set Voltage Limit / writable signals 0x3000 ¦ Status Flags ¦ 8 ¦ rx ¦ TRUE ¦ Bitfield status register (telemetry only)

## Sheet 3 — Variables ⭐ (most important)

Each row defines one numeric signal extracted from an incoming frame. The app reads bytes at the computed offset, applies scale and offset, and shows the result in the Data Table.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| id_or_address | hex(required) | 0x0010 | Frame ID (framed) or Modbus register address this signal lives in. If the ID is not declared in the Frames sheet, the loader auto-creates a frame namedNode 0x<id>— convenient for Modbus configs that skip the Frames sheet entirely. |
| signal_name | text(required) | Pack Voltage | Unique name shown in the Data Table. Must be unique within each frame. Forcount > 1the uniqueness check is appliedafterexpansion, soCell Vwithcount = 12becomes 12 distinct names. |
| data_type | enum(required) | uint16 | Raw data type in the payload:uint8,int8,uint16,int16,uint32,int32,float32,float64. Byte width is derived from the type and used to compute the next signal's start offset automatically. |
| count | integer | 12(default: 1) | Repeat count.count=12on a signal namedCell Vauto-createsCell V 1…Cell V 12at consecutive byte offsets — no copy-pasting 12 rows. Must be ≥ 1. |
| byte_order | enum | little(default) | little(LSB first, most embedded MCUs) orbig(MSB first, network order). Validated at load — any other value is a hard error. |
| scale | number | 0.001(default: 1.0) | Multiply the raw integer by this. E.g. raw = 12500, scale = 0.001 → displayed as 12.5. |
| offset | number | -40(default: 0.0) | Added after scaling. Formula:value = raw × scale + offset. E.g. temperature sensor with scale=0.1, offset=−40 converts 850 → 45 °C. |
| unit | text | V(blank ok) | Engineering unit shown in the Unit column (e.g.V,A,°C,%,rpm). |
| group | text | Cell Voltages(blank ok) | Group name used by CalcGroups for min/max/avg aggregation. Leave blank if not needed. |
| read_write | enum | R(default) | R= read-only.W= write-only.RW= readable and writable. OnlyW/RWsignals appear in the Parameter Editor. Value is upper-cased on load. |
| min_value | number | 0(blank = no limit) | Minimum valid write value. The Parameter Editor rejects inputs below this. Leave blank for no limit. |
| max_value | number | 5.0(blank = no limit) | Maximum valid write value. Leave blank for no limit. |
| description | text | Total pack voltage(blank ok) | Optional note — not displayed in the app. |
| enabled | TRUE/FALSE | TRUE(default) | SetFALSEto hide this signal from all panels without deleting the row — the row is skipped entirely at load time. |

ℹ Note Byte offsets are computed automatically. Signals within a frame are packed in the order they appear in the sheet. You do not enter a start byte — the loader calculates it from each signal's data_type size.

ℹ Note Frame auto-creation. If id_or_address refers to a frame ID that does not exist in the Frames sheet, the loader silently creates one named Node 0x<id>. This is the recommended workflow for Modbus configs — declare your registers in Variables only and omit the Frames sheet altogether.

Bundled template content (showing the key columns; read_write, min_value, max_value, description, enabled omitted for width):

id_or_address ¦ signal_name ¦ data_type ¦ count ¦ byte_order ¦ scale ¦ offset ¦ unit ¦ group 0x1000 ¦ Pack Voltage ¦ uint16 ¦ 1 ¦ little ¦ 0.01 ¦ 0 ¦ V ¦ 0x1000 ¦ Pack Current ¦ int16 ¦ 1 ¦ little ¦ 0.1 ¦ 0 ¦ A ¦ 0x1000 ¦ Pack SOC ¦ uint8 ¦ 1 ¦ little ¦ 1 ¦ 0 ¦ % ¦ 0x1000 ¦ Pack Temperature ¦ int16 ¦ 1 ¦ little ¦ 0.1 ¦ -40 ¦ °C ¦ 0x2000 ¦ Cell Voltage ¦ uint16 ¦ 8 ¦ little ¦ 0.001 ¦ 0 ¦ V ¦ Cell Voltages ← count=8 auto-expands to 8 signals 0x3000 ¦ Fault Flags ¦ uint8 ¦ 1 ¦ little ¦ 1 ¦ 0 ¦ ¦ ← see Bitfields sheet 0x3000 ¦ BMS State ¦ uint8 ¦ 1 ¦ little ¦ 1 ¦ 0 ¦ ¦ ← see Enums sheet 0x2000 ¦ Voltage Limit ¦ uint16 ¦ 1 ¦ little ¦ 0.01 ¦ 0 ¦ V ¦ ← read_write=RW → Parameter Editor

## Legacy — FrameConfig

An older schema with explicit start_byte and byte_length. Only used when the Variables sheet is absent. Use Variables for all new configs. Every column in this schema is required — there are no optional fields, no count auto-expansion, no enabled toggle, no per-frame Frames sheet (each row carries its own frame_name).

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| frame_id_hex | hex(required) | 0x10 | Frame this signal belongs to. Accepts0x10,10, or16(decimal).Note:the column header is literallyframe_id_hex, notframe_id— the loader will reject the sheet if this column is missing. |
| frame_name | text(required) | BMS Status | Display name for the frame. Repeated on every row of the sameframe_id_hex; the first occurrence is used to build the frame definition. |
| signal_name | text(required) | Voltage | Display name. Must be non-empty and unique within each frame — duplicates are a hard error at load time. |
| start_byte | integer(required) | 0 | Zero-based byte offset within the payload. Unlike the Variables sheet, offsets here arenotauto-computed; you must specify every signal's exact position. |
| byte_length | integer(required) | 2 | Number of bytes to read. Must be 1..8 — anything outside this range is a hard error. Fordata_type = float, must be exactly 4 or 8. |
| endianness | enum(required) | little | littleorbig. Validated at load — any other value is a hard error. |
| data_type | enum(required) | uint | int,uint, orfloat. Coarser than the Variables sheet'suint16/int32/etc. — width comes frombyte_length. |
| scale | number(required) | 0.01 | Multiplier applied to the raw value. Blank falls back to1.0; non-numeric is a hard error. |
| offset | number(required) | 0 | Addend applied after scaling. Blank falls back to0.0; non-numeric is a hard error. Formula:value = raw × scale + offset. |
| unit | text(required) | V | Display unit shown in the Unit column. May be an empty string but the column header itself must be present. |

⚠ Warning Common pitfall: if your legacy CSV uses frame_id instead of frame_id_hex as the column header, the loader will reject the entire config with missing required columns. Rename the column to frame_id_hex or migrate to the modern Variables sheet.

Bundled template content (legacy schema for the same 0x1000 frame — every byte offset and length is explicit):

frame_id_hex ¦ frame_name ¦ signal_name ¦ start_byte ¦ byte_length ¦ endianness ¦ data_type ¦ scale ¦ offset ¦ unit 0x1000 ¦ BMS Status ¦ Pack Voltage ¦ 0 ¦ 2 ¦ little ¦ uint ¦ 0.01 ¦ 0 ¦ V 0x1000 ¦ BMS Status ¦ Pack Current ¦ 2 ¦ 2 ¦ little ¦ int ¦ 0.1 ¦ 0 ¦ A 0x1000 ¦ BMS Status ¦ Pack SOC ¦ 4 ¦ 1 ¦ little ¦ uint ¦ 1 ¦ 0 ¦ % 0x1000 ¦ BMS Status ¦ Pack Temperature ¦ 5 ¦ 2 ¦ little ¦ int ¦ 0.1 ¦ -40 ¦ °C

## Sheet 4 — Bitfields

Breaks a single integer signal into named Boolean flags. Each bit appears as an ON/OFF indicator in the Bitfields panel.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| id_or_address | hex(required) | 0x0020 | Frame ID of the parent signal. |
| signal_name | text(required) | Fault Flags | Must exactly match a signal name in the Variables sheet. If the(id_or_address, signal_name)pair does not exist, the loader rejects the entire config at load time withunknown variable. |
| bit_index | integer(required) | 0 | Which bit to extract from the signal's raw integer value. Decoded asraw & (1 << bit_index), so bit 0 is the least-significant bit, bit 7 the MSB of auint8, bit 15 the MSB of auint16, etc. Valid range follows the parent signal's width (0..7 foruint8, 0..15 foruint16, 0..31 foruint32). |
| label | text(required) | Overvoltage | Name shown in the Bitfields panel for this bit. |
| active_text | text | FAULT(default: ON) | Text shown when the bit is 1. Blank cells also fall back toON. |
| inactive_text | text | OK(default: OFF) | Text shown when the bit is 0. Blank cells also fall back toOFF. |

ℹ Note Bit numbering recap. Bitfield rows operate on the fully assembled raw integer of the parent signal (after byte-order is applied), not on raw payload bytes. A uint16 read as little-endian is reconstructed into a single 16-bit integer first; bit_index = 15 then picks its most-significant bit regardless of which physical byte that bit came from.

Bundled template content (decodes the 8-bit Fault Flags register from frame 0x3000):

id_or_address ¦ signal_name ¦ bit_index ¦ label ¦ active_text ¦ inactive_text 0x3000 ¦ Fault Flags ¦ 0 ¦ Charging ¦ YES ¦ NO 0x3000 ¦ Fault Flags ¦ 1 ¦ Discharging ¦ YES ¦ NO 0x3000 ¦ Fault Flags ¦ 2 ¦ Balancing ¦ YES ¦ NO 0x3000 ¦ Fault Flags ¦ 3 ¦ Fault ¦ FAULT ¦ OK 0x3000 ¦ Fault Flags ¦ 4 ¦ Overvoltage ¦ FAULT ¦ OK 0x3000 ¦ Fault Flags ¦ 5 ¦ Undervoltage ¦ FAULT ¦ OK 0x3000 ¦ Fault Flags ¦ 6 ¦ Overtemp ¦ FAULT ¦ OK 0x3000 ¦ Fault Flags ¦ 7 ¦ Ready ¦ YES ¦ NO

## Sheet 5 — Enums

Maps raw integer values of a signal to readable text labels. Results appear in the Enums panel and replace the raw number in the Data Table's Value column.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| id_or_address | hex(required) | 0x0010 | Frame ID of the parent signal. |
| signal_name | text(required) | State | Must exactly match a signal name in Variables. If the(id_or_address, signal_name)pair does not exist, the loader rejects the entire config at load time withunknown variable. |
| value | integer(required) | 2 | Raw integer value from the hardware. Decimal (2) or hex with prefix (0x02) — both work viaint(value, 0). |
| label | text(required) | RUN | Human-readable text shown when the signal equals this value. Empty labels are allowed but render as blank in the Enums panel. |

Bundled template content (decodes the BMS State signal from frame 0x3000 into one of five labels):

id_or_address ¦ signal_name ¦ value ¦ label 0x3000 ¦ BMS State ¦ 0 ¦ Idle 0x3000 ¦ BMS State ¦ 1 ¦ Charging 0x3000 ¦ BMS State ¦ 2 ¦ Discharging 0x3000 ¦ BMS State ¦ 3 ¦ Fault 0x3000 ¦ BMS State ¦ 4 ¦ Service

ℹ Note Duplicate values overwrite silently. Enum mappings are stored as {value → label} per signal, so if two rows share the same (id_or_address, signal_name, value) the second row's label wins. No warning is emitted — keep your value column unique within each signal.

## Sheet 6 — CalcGroups

Computes aggregate statistics across decoded signals sharing the same group name. Results appear as extra rows at the bottom of the Data Table.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| group_name | text(required) | Cell Voltages | Must exactly match a non-emptygroupvalue in Variables. If the group does not exist, the loader rejects the entire config at load time withunknown group. |
| operations | pipe-list(required) | min¦max¦avg¦diff | Statistics to compute. Supported values aremin,max,avg,sum, anddiff(max − min). Separate with¦; tokens are trimmed and lowercased. Any unsupported value rejects the config. |
| unit | text | V(blank ok) | Unit shown for the calculated rows. |
| frame_id | integer / hex | 0x0010(blank = no frame filter) | Optional frame filter. Set this to run the calculation only for one frame; leave blank to run it for each decoded frame that contains signals in the group. |
| enabled | TRUE/FALSE | TRUE(default: TRUE) | SetFALSEto disable without deleting the row. Disabled rows are parsed, then skipped at load time. |

⚠ Warning Frame scope pitfall. A blank frame_id does not combine values across multiple frames into one global statistic. It means "no frame filter": the calculation runs separately for each decoded frame that has signals in the matching group. Use a 0x prefix for hex frame IDs here; bare values such as 10 are parsed as decimal.

Bundled template content (computes 4 stats across the 8 expanded Cell Voltage 1..8 signals from frame 0x2000):

group_name ¦ operations ¦ unit ¦ frame_id ¦ enabled Cell Voltages ¦ min¦max¦diff¦avg ¦ V ¦ 0x2000 ¦ TRUE

How this links up:

1.  In Variables, the Cell Voltage row at frame 0x2000 has count = 8 and group = Cell Voltages — auto-expanded into Cell Voltage 1 … Cell Voltage 8.

2.  This CalcGroups row picks them up and adds four extra rows to the Data Table — Cell Voltages min, max, avg, diff — updated live with each 0x2000 frame.

## Sheet 7 — TxCommands

Each enabled row creates one entry in the TX Commands panel. The app builds a packet from the static payload plus any matching TxCommandFields, then sends it to your hardware when you click Send.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| command_name | text(required) | Reset Faults | Command label shown in the TX Commands selector. Matching rows in TxCommandFields attach user-input fields to this command. |
| id_or_address | hex(required) | 0x0090 | Frame ID (or Modbus register address) of the outgoing packet. Must be non-empty; use a0xprefix for clarity. |
| payload_hex | hex bytes | 01 00(blank ok) | Static payload bytes prepended before any TxCommandFields values. Leave blank if all payload bytes come from user-input fields. Invalid hex is reported when the command is previewed or sent. |
| description | text | Clears all active fault flags(blank ok) | Free-text note shown as the tooltip on the TX Commands selector — hover the currently selected command (or any entry in the open dropdown) to see this text. Keep it concise; one line is ideal. |
| enabled | TRUE/FALSE | TRUE(default: TRUE) | SetFALSEto hide the command without deleting the row. Disabled rows are skipped at load time. |

⚠ Warning Duplicate names overwrite silently. Commands are stored by command_name, so if two enabled rows use the same name, the later row wins and no warning is emitted. Keep command_name unique.

ℹ Note Reusing Addresses for Telemetry and Commands: Yes, you can (and often should) use the same id_or_address (frame ID or Modbus register address) in both the Variables sheet (Sheet 3) and the TxCommands sheet (Sheet 7). In a custom framed protocol, telemetry variables you read from the device can share a frame ID with commands you write to the device. The device's firmware parses the frame ID to decide how to process it.In a Modbus RTU protocol, a register address (like 0x1000) represents a specific physical memory location on the slave device. You read its value by defining it in the Variables sheet and setting a poll interval in the PollingSchedule sheet. You write a value to it by defining a command with the same address in the TxCommands sheet. Make sure that if you define a frame in the Frames sheet (Sheet 2) that is used for both RX and TX, its direction column is set to rxtx (which is the default). If it is set to rx (receive-only), the app will hide any TX commands using that frame ID to prevent accidental writes.

Bundled template content (one static-payload command and one parameterized command — the latter has matching rows in TxCommandFields):

command_name ¦ id_or_address ¦ payload_hex ¦ description ¦ enabled Reset Faults ¦ 0x1000 ¦ FF FF ¦ Clears all active fault flags on the BMS ¦ TRUE Set Voltage Limit ¦ 0x2000 ¦ ¦ Set the overvoltage threshold (requires field input) ¦ TRUE

## Sheet 8 — TxCommandFields

Adds user-input numeric fields to a TX command. When a command has fields, the TX Commands panel shows an input form. The app encodes, scales, and appends those values after the command's static payload.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| command_name | text(required) | Set Current Limit | Links this field to a TxCommand. Must match an enabled TxCommandscommand_nameexactly to appear in the form. |
| signal_name | text(required) | Current Limit | Label shown next to the input field. Keep this unique within a command so each input has its own value. |
| data_type | enum(required) | uint16 | How the value is encoded in the outgoing packet:uint8,int8,uint16,int16,uint32,int32,float32, orfloat64. Value is lowercased on load; unsupported types reject the config. |
| byte_order | enum | little(default) | littleorbigendian. Blank cells default tolittle; any other value rejects the config. |
| scale | number | 0.1(default: 1.0) | Encoding formula:raw = (user_value − offset) / scale. E.g. user enters 50.0 A with scale=0.1 → raw = 500. A zero scale loads successfully but the command cannot be built or sent. |
| offset | number | 0(default: 0.0) | Subtracted from the user value before dividing by scale. |
| unit | text | A(blank ok) | Shown next to the input field. |
| min_value | number | 0(blank = no limit) | App refuses to transmit if the entered value is below this. |
| max_value | number | 200(blank = no limit) | App refuses to transmit if the entered value is above this. |
| default | number | 100(blank ok) | Pre-filled value shown in the input box when the command form opens. Leave blank to require the user to enter a value before previewing or sending. |

⚠ Warning Field matching is silent. Rows whose command_name does not match an enabled TxCommands row are parsed but never shown. If two fields on the same command reuse the same signal_name, the form labels collide and the later input value is reused for that name. Keep command_name valid and signal_name unique within each command.

Bundled template content (attaches one user-entered voltage field to the Set Voltage Limit command — encoded as uint16 little with scale 0.01):

command_name ¦ signal_name ¦ data_type ¦ byte_order ¦ scale ¦ offset ¦ unit ¦ min_value ¦ max_value ¦ default Set Voltage Limit ¦ Voltage Limit (V) ¦ uint16 ¦ little ¦ 0.01 ¦ 0 ¦ V ¦ 0 ¦ 60 ¦ 55

A user typing 55.5 in the form sends raw = (55.5 − 0) / 0.01 = 5550 appended after the static payload (which is empty in this case).

## Sheet 9 — PollingSchedule

Only needed if your hardware uses a request / response model — i.e. it only sends data when asked. Each row tells the app to periodically send a request packet and wait for a response.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| id_or_address | hex(required) | 0x0010 | Frame ID (or Modbus register address) to request. Must be non-empty; bare values such as0010are treated as hex, so use a0xprefix for clarity. |
| interval_ms | integer(required) | 100 | How often to send the request, in milliseconds. Parsed as an integer and used to schedule the next poll after each request completes. |
| timeout_ms | integer(required) | 50 | How long to wait for the matching response before marking the request as timed out. Parsed as an integer; the current loader reads this column directly, so the header and value must be present. |
| enabled | TRUE/FALSE | TRUE(default: TRUE) | SetFALSEto pause polling for this frame without deleting the row. Disabled rows are skipped at load time. |

ℹ Note Polling starts only when Auto-Fetch is enabled. Requests are sent round-robin across enabled targets; if a request cannot be built for the configured protocol, that target is disabled and reported once.

⚠ Warning If your hardware streams data automatically without being asked (e.g. sends packets every 50 ms on its own), leave this sheet empty and do not use Auto-Fetch. Polling an already-streaming device sends unnecessary requests and may corrupt the data stream.

Bundled template content (polls all three telemetry frames with per-frame intervals and timeouts):

id_or_address ¦ interval_ms ¦ timeout_ms ¦ enabled 0x1000 ¦ 100 ¦ 50 ¦ true 0x2000 ¦ 500 ¦ 100 ¦ true 0x3000 ¦ 200 ¦ 50 ¦ true

## Sheet 10 — SerialDefaults

Config-driven defaults for the Connection dialog. When the dialog opens, each field is filled from QSettings (your last-used choice) if present, otherwise from this sheet. That means a fresh install — or a freshly loaded config you've never connected with before — opens the Connect dialog already showing this config's expected baud rate, parity, etc.; subsequent runs keep whatever you explicitly chose.

| Column | Type | Example | What it does |
| --- | --- | --- | --- |
| baud_rate | integer(required) | 115200 | Baud-rate default for the Connection dialog. Used on first open; thereafter QSettings wins. |
| data_bits | integer(required) | 8 | Data-bits default for the Connection dialog. Combo offers8and7. |
| stop_bits | number(required) | 1(blank: 1.0) | Stop-bits default for the Connection dialog. Combo offers1,1.5,2. Blank cells fall back to1.0. |
| parity | text(required) | N(blank: N) | Parity default for the Connection dialog. Combo offersN,E,O. Blank cells fall back toN; values are uppercased on load. |
| timeout_ms | integer(required) | 100 | Read-timeout default for the Connection dialog. Combo offers20/50/100/250/500/1000ms. |

⚠ Warning Do not add a partially blank row. If this sheet is absent or empty, the loader uses internal defaults. If the sheet contains a row, all five column headers are required; baud_rate, data_bits, and timeout_ms must contain valid integers.

Bundled template content (standard 8-N-1 at 115 200 baud with a 100 ms read timeout — matches what the Connection dialog uses by default):

baud_rate ¦ data_bits ¦ stop_bits ¦ parity ¦ timeout_ms 115200 ¦ 8 ¦ 1 ¦ N ¦ 100

# 6. Appearance & Updates

## Switching Light / Dark Theme

Open View → Theme and pick one of:

•  Dark — slate background with light text. Best for long monitoring sessions and dim labs.

•  Light — white background with dark text. Best for outdoor / brightly lit environments and screenshots in reports.

•  System — asks Qt to follow the operating system's light/dark preference.

The theme switch is instant. The app reapplies its window styling, rebuilds toolbar/menu icons, retints the live plot canvas, updates the Analysis Suite and the X-Y Plotter windows if open, and remembers your choice between launches.

ℹ Note Toolbar and menu icons are theme-aware. In light theme they are drawn with a dark glyph so they remain readable on the white menu background; in dark theme they switch to a light glyph automatically. No restart needed.

⚠ Warning System theme caveat. The base Qt palette follows the OS setting, but Bytehound's custom plot, icon, and Windows title-bar tinting are tuned for the explicit Dark and Light choices. If anything looks inconsistent in System, choose Dark or Light directly.

## Check for Updates

Open Help → Check for Updates. The app will:

1.  Read the local version.json, contact its configured update manifest over HTTPS, and compare the published version against your installed version.

2.  If you are already on the latest build, show a brief You are on the latest version confirmation.

3.  If a newer version is available, show the version number and release notes and ask whether you want to install it.

4.  On confirmation, download the installer to your system temp folder with a progress dialog. You can cancel while the download is running — the partial file is automatically cleaned up.

5.  Verify the installer SHA-256 checksum against the manifest before launching. If the digest does not match, or if the manifest does not provide a checksum, the install is refused; mismatched or partial downloads are deleted.

6.  After the download is verified, ask one more time before launching the installer in silent mode. The app then exits so the installer can replace the running executable. When the installer finishes, relaunch Bytehound manually.

ℹ Note Update checks are entirely on-demand. The app never contacts the network on its own — nothing is downloaded or installed without your explicit click.

⚠ Warning If you see "Failed to check for updates: connection timed out", the manifest server is unreachable from this machine. Check your firewall / proxy settings, or simply skip the update — the app continues to work normally.

## Diagnostics & Troubleshooting

If you encounter issues, use Help → Copy Diagnostics. This generates a complete snapshot of your session state—including OS details, Bytehound version, Qt/Python versions, active config path, connection status, packet counters, and the tail of the internal application log. You can paste this directly into a bug report to provide all necessary context in one step without needing to manually hunt down the log file.

# 7. Application Limitations & Safety Caveats

Bytehound is designed to be highly flexible and performant. However, when using the application for real-time telemetry and control, you must keep the following design constraints and safety features in mind:

## Performance & Packet Processing Rates

•  Maximum Frame Rate Limit: The Qt GUI telemetry pipeline is optimized and verified to handle packet rates up to 1000 Hz (1 kHz). Above this rate, packet processing and UI redraw operations may consume significant CPU resources.

•  Queue Saturation (⚠️ Queue Saturated): If incoming serial data rates exceed the system's capacity, or if the OS stalls the GUI thread (e.g., during window dragging), the serial packet queue or log flush queue may saturate. When this happens, a yellow warning badge appears in the bottom status bar, and newer packets/logs will be dropped to protect system memory. Click the badge to dismiss it. If it reappears, the underlying condition persists — reduce the incoming data rate, increase the flush interval under Tools → Logging Settings, or close unused dock panels to free up the GUI thread.

•  Odd Byte Widths: Decoded signal widths of 3, 5, 6, and 7 bytes are optimized using zero-copy struct.unpack_from combined with bitwise shifting and masking, eliminating the performance penalty of slicing and matching power-of-two decoding performance.

## Data Logging & Buffer Safety

•  Decoded Log Crash Safety: The Decoded Excel log (*_decoded.xlsx) is written incrementally to temporary files (.tmp_data and .tmp_meta) during the session and compiled at the end. If the application crashes, the unsaved data is not lost; Bytehound automatically scans and prompts to recover these logs on the next launch.

•  Plot History Limits: Plot history is stored in a fixed-capacity ring buffer (default 6,000 samples per signal). In long monitoring sessions, older points will wrap around and be discarded from memory to prevent memory leaks. Consequently, scrolling back to the beginning of a multi-hour test in the Live Plot is not supported; use the Analysis Suite on the saved log files instead.

•  Raw Console and Log line limits: The Raw Console is capped at 3,000 blocks/lines of history, and the Activity Log is capped at 5,000 blocks. Old messages are automatically truncated from the display buffer.

## Dock Window CPU Optimization (Data Gaps)

•  Hidden Dock Optimization: To save CPU cycles on low-spec hardware, the application suspends UI rendering operations on hidden panels. However, plot history buffering remains fully active in the background for closed panels, ensuring that reopening the Live Plot displays a continuous timeline with no data gaps.

## Configuration Constraints

•  Strict Config Duplicate Validations: The configuration loader validates all sheets for duplicates. If you define duplicate entries—such as duplicate variables in a frame, duplicate tx_commands, duplicate enums values, duplicate bit_index/labels on the same bitfield signal, duplicate calculations, or duplicate polling schedules—the loader will raise a validation ConfigError rather than silently overwriting.

•  Modbus RTU Specifications: The Modbus RTU protocol specifies big-endian encoding for register addresses and data values, and little-endian encoding for the CRC checksum. Custom byte_order configurations in the Protocol sheet are ignored when parser_type = modbus_rtu is selected.

•  Polling vs. Streaming: Do not enable Auto-Fetch (F10) on a device that is already streaming data automatically (unsolicited). Sending poll requests to a streaming device will cause transmission collisions and corrupt the incoming data packets.